Introduction to Sensor Networks

- Constraints and Challenges

- Limited hardware:

  - Storage

  - Processing

  - Communication

  - Energy supply (battery power)

- Limited support for networking:

  - Peer-to-peer network

  - Unreliable communication

  - Dynamically changing

- Limited support for software development:

  - Real-time tasks that involve dynamic collaboration among nodes

  - Software architecture needs to be co-designed with the information processing architecture

- Sensor vs other ad hoc networks

Sensor networks

- Special communication patterns: many-to-one, one-to-many, attribute-based

- Static sensors in many applications

- Constraints more severe than general ad hoc networks

- Distributed collaborative computing

Mobile ad hoc networks

- General-purpose communication involving mobile devices

- Devices are often mobile

- Applications

- Environmental monitoring

  - Traffic, habitat, security

- Industrial sensing and diagnostics

  - Manufacturing, supply chains

- Context-aware computing

  - Intelligent homes

- Military applications:

  - Multi-target tracking

- Infrastructure protection:

  - Power grids

- Why are Sensor Networks Special?

- Matchbox-sized to Shoebox-sized nodes

  - MICA motes: 4 MHz, 32 KB, 15 kJ, 30 m

  - Sensoria sensor: 400 MHz, 32 MB, 300 kJ, 100 m

- More severe power constraints than PDAs, mobile phones, laptops

- Mobility may be limited, but failure rate higher

- Usually under one administrative control

- A sensor network gathers and processes specific kinds of data relevant to application

- Potentially large-scale networks comprising of thousands of tiny sensor nodes

- Advantages of networked sensing

- Detection:

  - Improved signal-to-noise ratio by reducing average distance between source and sensor

- Energy:

  - A path with many short hops has less energy consumption than a path with few long hops

- Robustness:

  - Address individual sensor node or link failures

- Focus Problems

- Medium-access and power control:

  - Power saving techniques integral to most sensor networks

  - Possibility of greater coordination among sensor nodes to manage channel access

- Routing protocols:

  - Geographic routing and localization

  - Attribute-based routing

  - Energy-Awareness

- Synchronization protocols:

  - Many MAC and application level protocols rely on synchronization

- Query and stream processing:

  - Sensor network as a database

  - Streams of data being generated at the nodes by their sensors

  - Need effective in-network processing and networking support

- MAC Protocols for Sensor Networks

- Contention-Based:

  - Random access protocols

  - IEEE 802.11 with power saving methods

- Scheduling-Based:

  - Assign transmission schedules (sleep/awake patterns) to each node

  - Variants of TDMA

- Hybrid schemes

- Proposed MAC Protocols

- PAMAS [SR98]:

  - Contention-based access

  - Powers off nodes that are not receiving or forwarding packets

  - Uses a separate signaling channel

- S-MAC [YHE02]:

  - Contention-based access

- TRAMA [ROGLA03]:

  - Schedule- and contention-based access

- Wave scheduling [TYD+04]:

  - Schedule- and contention-based access

- S-MAC

- Identifies sources of energy waste [YHE03]:

  - Collision

  - Overhearing

  - Overhead due to control traffic

  - Idle listening

- Trade off latency and fairness for reducing energy consumption

- Components of S-MAC:

  - A periodic sleep and listen pattern for each node

  - Collision and overhearing avoidance

- S-MAC: Sleep and Listen Schedules

- Each node has a sleep and listen schedule and maintains a table of schedules of neighboring nodes

- Before selecting a schedule, node listens for a period of time:

  - If it hears a schedule broadcast, then it adopts that schedule and rebroadcasts it after a random delay

  - Otherwise, it selects a schedule and broadcasts it

- If a node receives a different schedule after selecting its schedule, it adopts both schedules

- Need significant degree of synchronization

- S-MAC: Collision and Overhearing Avoidance

- Collision avoidance:

  - Within a listen phase, senders contending to send messages to same receiver use 802.11

- Overhearing avoidance:

  - When a node hears an RTS or CTS packet, then it goes to sleep

  - All neighbors of a sender and the receiver sleep until the current transmission is over

- TRAMA

- Traffic-adaptive medium adaptive protocol [ROGLA03]

- Nodes synchronize with one another

  - Need tight synchronization

- For each time slot, each node computes an MD5 hash, that computes its priority

- Each node is aware of its 2-hop neighborhood

- With this information, each node can compute the slots it has the highest priority within its 2-hop neighborhood

- TRAMA: Medium Access

- Alternates between random and scheduled access

- Random access:

  - Nodes transmit by selecting a slot randomly

  - Nodes can only join during random access periods

- Scheduled access:

  - Each node computes a schedule of slots (and intended receivers) in which will transmit

  - This schedule is broadcast to neighbors

  - A free slot can be taken over by a node that needs extra slots to transmit, based on priority in that slot

  - Each node can determine which slots it needs to stay awake for reception

- Wave Scheduling

- Motivation:

  - Trade off latency for reduced energy consumption

  - Focus on static scenarios

- In S-MAC and TRAMA, nodes exchange local schedules

- Instead, adopt a global schedule in which data flows along horizontal and vertical “waves”

- Idea:

  - Organize the nodes according to a grid

  - Within each cell, run a leader election algorithm to periodically elect a representative (e.g., GAF [XHE01])

  - Schedule leaders’ wakeup times according to positions in the grid

- Wave Scheduling: A Simple Wave

- Wave Scheduling: A Pipelined Wave

- Wave Scheduling: Message Delivery

- When an edge is scheduled:

  - Both sender and receiver are awake

  - Sender sends messages for the duration of the awake phase

  - If sender has no messages to send, it sends an NTS message (Nothing-To-Send), and both nodes revert to sleep mode

- Given the global schedule, route selection is easy

  - Depends on optimization measure of interest

  - Minimizing total energy consumption requires use of shortest paths

  - Minimizing latency requires a (slightly) more complex shortest-paths calculation

- Routing strategies

- Geographic routing

  - Greedy routing

  - Perimeter or face routing

  - Localization and location services

- Attribute-based routing:

  - Directed diffusion

  - Rumor routing

  - Geographic hash tables

- Energy-aware routing:

  - Minimum-energy broadcast

  - Energy-aware routing to a region

- Localization techniques

- Methods that allow the nodes in a network to determine their geographic positions

- Use of current GPS systems not feasible:

  - Cost

  - Power consumption

  - Form factor

  - Do not work indoors or under dense foliage

- Sensor network approach:

  - A small number of nodes are equipped with GPS receivers and can localize themselves

  - Other nodes localize themselves using landmarks

- Two ranging techniques:

  - Received signal strength (RSS)

  - Time of arrival (TOA/TDOA)

- Received signal strength (RSS)

- If the source signal strength and attenuation are known, then receiver can estimate its distance from the sender

- Unfortunately, RSS (α) can vary substantially:

  - Fading

  - Multipath effects

- Apparently, localization to within meters is the best one can do with RSS in practice

  - Time of arrival

- Basic idea:

  - Measure time it takes for a signal to travel from sender to receiver

  - Multiply by signal propagation speed

- Need sender and receiver to be synchronized, and exact time of transmission known

- Exact transmission time hard to determine

- Time Difference of Arrival (TDOA)

  - Measure TDOA at two receivers

  - Can obtain difference in distances between the two receivers and the sender

- Signal speed not necessarily constant

  - Local beacons and measurements to estimate signal speed

- Apparently distance measurement to within centimeters achievable

- Localization using ranging

- Obtain multiple distance measurements using multiple landmarks

- Write out equations, including measurement errors

  - Variables are the errors and the location coordinates

- Minimize the weighted total squared error to yield the desired estimate

- [SHS01]

- Geographic Location Service (GLS)

- Use sensor nodes as location servers

- Organize the space as a hierarchical grid according to a spatial quad-tree

- Each node has a unique ID (e.g., MAC address)

- Location servers for a node X:

  - One server per every vertex of the quad tree that is a sibling of a vertex that contains X

  - This server is the node with smallest ID larger than X in the region represented by the quad tree vertex (with wraparound, if necessary)

- To locate a node X:

  - Traverse up the quad tree repeatedly seeking the node that has the smallest ID larger than X

- [L+00]

- Performance of GLS

- Depth of the quad-tree = O(log n), where n is the number of nodes in the network

- Therefore, number of location servers for a given node is O(log n)

- How many nodes does a given node serve?

  - If the Ids are randomly distributed, can argue that the expected number of nodes served per node is O(log n)

  - Even with high probability

- If the source and destination lie in a common quad-tree tile at level i

  - At most i steps are needed to reach the location server for the destination

  - Cost of location service distance-sensitive

- Attribute-based routing

- Data-centric approach:

  - Not interested in routing to a particular node or a particular location

  - Nodes desiring some information need to find nodes that have that information

- Attribute-value event record, and associated query

  - Directed diffusion

- Sinks: nodes requesting information

- Sources: nodes generating information

- Interests: records indicating

  - A desire for certain types of information

  - Frequency with which information desired

- Key assumption:

  - Persistence of interests

- Approach:

  - Learn good paths between sources and sinks

  - Amortize the cost of finding the paths over period of use

- [IGE00]

- Diffusion of interests and gradients

- Interests diffuse from the sinks through the sensor network

- Nodes track unexpired interests

- Each node maintains an interest cache

- Each cache entry has a gradient

  - Derived from the frequency with which a sink requests repeated data about an interest

  - Sink can modify gradients (increase or decrease) depending on response from neighbors

  - Rumor routing

- Spread information from source and query from sink until they meet

- Source information and query both follow a one-dimensional strategy

  - Random walk

  - Straight ray emanating from origin

- As agents move, both data and interest are stored at intermediate nodes

- Query answered at intersection point of these two trajectories

- Multiple sources and sinks:

  - Merge interest requests and data

- [BE02]

- Geographic hash tables (GHT)

- Hash attributes to specific geographic locations in the network

  - Information records satisfying the attributes stored at nodes near location

- Every node is aware of the hash function

  - Query about records satisfying a given attribute are routed to the relevant location

- Load balancing achieved by hash function

- Information also stored locally where it is generated

- Can also provide replication through a hierarchical scheme similar to GLS

- [R+03]

- GHT & geographic routing

- The nodes associated with a particular attribute are the ones that form the perimeter around the hashed location

- One of these is selected as a home node

  - Node closest to the location

  - Determined by going through the cycle

  - Recomputed periodically to allow for changes

- Energy-aware routing

- Need energy-efficient paths

- Notions of energy-efficiency:

  - Select path with smallest energy consumption

  - Select paths so that network lifetime is maximized

    - When network gets disconnected

    - When one node dies

    - When area being sensed is not covered any more

- Approaches:

  - Combine geographic routing with energy-awareness

  - Minimum-energy broadcast

  - Geography and energy-awareness

- Recall that geographic routing is divided into two steps:

  - Planarization of the underlying transmission graph

  - Routing on the planar graph using greedy and perimeter routing

- Can we obtain planar subgraphs that contain energy-efficient paths?

  - Gabriel graph and similar variants do not suffice

  - Planar subgraphs based on Delaunay triangulation have desired properties

  - Unfortunately, not necessary that greedy and perimeter routing will find such paths

- Energy available at nodes dynamically changes as different paths are used

- Incorporating residual node energy

- Cost for each edge has two components:

  - Energy consumption for transmission

  - Energy already consumed at each endpoint

  - A suitable weighted combination of both

- Geographic Energy-Aware Routing (GEAR)

  - If neighbors exist that are closer with respect to both distance and cost, select such a node that has smallest cost

  - Otherwise, select a node that has smallest cost

  - Costs updated as energy of nodes change

- [YGE01]

- Minimum Energy Broadcast Routing

- Given a set of nodes in the plane

- Goal: Broadcast from a source to all nodes

- In a single step, a node may broadcast within a range by appropriately adjusting transmit power

- Minimum Energy Broadcast Routing

- Energy consumed by a broadcast over range    is proportional to

- Problem: Compute the sequence of broadcast steps that consume minimum total energy

- Centralized solutions

- NP-complete [ZHE02]

- Three Greedy Heuristics

- In each tree, power for each node proportional to   th exponent of distance to farthest child in tree

- Shortest Paths Tree (SPT) [WNE00]

- Minimum Spanning Tree (MST) [WNE00]

- Broadcasting Incremental Power (BIP) [WNE00]

  - “Node” version of Dijkstra’s SPT algorithm

  - Maintains an arborescence rooted at source

  - In each step, add a node that can be reached with minimum increment in total cost

- SPT is       -approximate, MST and BIP have approximation ratio of at most 12 [WCLF01]

- Lower Bound on SPT

- Assume               nodes per ring

- Total energy of SPT:

- Optimal solution:

  - Broadcast to all nodes

  - Cost 1

- Approximation ratio

- Performance of the MST Heuristic

- Weight of an edge   	    equals

- MST for these weights same as Euclidean MST

  - Weight is an increasing function of distance

  - Follows from correctness of Prim’s algorithm

- Upper bound on total MST weight

- Lower bound on optimal broadcast tree

- Weight of Euclidean MST

- What is the best upper bound on the weight of an MST of points located in a unit disk?

  - In [6,12]!

- Structural Properties of MST

- Upper Bound on Weight of MST

- Assume    = 2

- For each edge  , its diamond accounts for an area of

- Lower Bound on Optimal

- For a non-leaf node   , let     denote the distance to farthest child

- Total cost is

- Performance of the BIP Heuristic

- Let                  be the nodes added in order by BIP

- Let     be the complete graph over the same nodes with the following weights:

  - Weight of edge               equals incremental power needed to connect

  - Weight of remaining edges same as in original graph

- MST of     same as BIP tree

- Synchronization in Sensor Networks

- Need for Synchronization in Sensor Networks

- Localization

  - Time of arrival methods require tight time synchronization between sender and receiver or among multiple receivers

- Coordinated actuation

  - Multiple sensors in a local area make a measurement

  - Determining the direction of a moving car through measurements at multiple sensors

- At the MAC level:

  - Power-saving duty cycling

  - TDMA scheduling

- Synchronization in Distributed Systems

- Well-studied problem in distributed computing

- Network Time Protocol (NTP) for Internet clock synchronization [Mil94]

- Differences: For sensor networks

  - Time synchronization requirements more stringent (μs instead of ms)

  - Power limitations constrain resources

  - May not have easy access to synchronized global clocks

  - NTP assumes that pairs of nodes are constantly connected and experience consistent communication delays

  - Often, local synchronization sufficient

- Network Time Protocol (NTP)

- Primary servers (S1) synchronize to national time standards

  - Satellite, radio, modem

- Secondary servers (S2, …) synchronize to primary servers and other secondary servers

  - Hierarchical subnet

- Measures of Interest

- Stability: How well a clock can maintain its frequency

- Accuracy: How well it compares with some standard

- Precision: How precisely can time be indicated

- Relative measures:

  - Offset (bias): Difference between times of two clocks

  - Skew: Difference between frequencies of two clocks

- Synchronization Between Two Nodes

- A sends a message to B; B sends an ack back

- A calculates clock drift and synchronizes accordingly

- Sources of Synchronization Error

- Non-determinism of processing times

- Send time:

  - Time spent by the sender to construct packet; application to MAC

- Access time:

  - Time taken for the transmitter to acquire the channel and exchange any preamble (RTS/CTS): MAC

- Transmission time: MAC to physical

- Propagation time: physical

- Reception time: Physical to MAC

- Receive time:

  - Time spent by the receiver to reconstruct the packet; MAC to application

- Sources of Synchronization Error

- Sender time = send time + access time + transmission time

  - Send time variable due to software delays at the application layer

  - Access time variable due to unpredictable contention

- Receiver time = receive time + reception time

  - Reception time variable due to software delays at the application layer

- Propagation time dependent on sender-receiver distance

  - Absolute value is negligible when compared to other sources of packet delay

  - If node locations are known, these times can be explicitly accounted

- Error Analysis

- Two Approaches to Synchronization

- Sender-receiver:

  - Classical method, initiated by the sender

  - Sender synchronizes to the receiver

  - Used in NTP

  - Timing-sync Protocol for Sensor Networks (TPSN) [GKS03]

- Receiver-based:

  - Takes advantage of broadcast facility

  - Two receivers synchronize with each other based on the reception times of a reference broadcast

  - Reference Broadcast Synchronization (RBS) [EGE02]

- TPSN

- Time stamping done at the MAC layer

  - Eliminates send, access, and receive time errors

- Creates a hierarchical topology

- Level discovery:

  - Each node assigned a level through a broadcast

- Synchronization:

  - Level i node synchronizes to a neighboring level i-1 node using the sender-receiver procedure

- Reference Broadcast Synchronization

- Motivation:

  - Receiver time errors are significantly smaller than sender time errors

  - Propagation time errors are negligible

  - The wireless sensor world allows for broadcast capabilities

- Main idea:

  - A reference source broadcasts to multiple receivers (the nodes that want to synchronize with one another)

  - Eliminates sender time and access time errors

  - Reference Broadcast Synchronization

- Simple form of RBS:

  - A source broadcasts a reference packet to all receivers

  - Each receiver records the time when the packet is received

  - The receivers exchange their observations

- Reference Broadcast Synchronization

- Clock skew:

  - Averaging assumes      equals 1

  - Find the best fit line using least squares linear regression

  - Determines       and

- Query Processing in Sensor Networks

- The Sensor Network as a Database

- From the point of view of the user, the sensor network generates data of interest to the user

- Need to provide the abstraction of a database

  - High-level interfaces for users to collect and process continuous data streams

- TinyDB [MFHH03], Cougar [YG03]

  - Users specify queries in a declarative language (SQL-like) through a small number of gateways

  - Query flooded to the network nodes

  - Responses from nodes sent to the gateway through a routing tree, to allow in-network processing

  - Especially targeted for aggregation queries

- Directed diffusion [IGE00]

  - Data-centric routing: Queries routed to specific nodes based on nature of data requested

- Challenges in Sensor Network Databases

- A potentially highly dynamic environment

- Relational tables are not static

  - Append-only streams where useful reordering operations too expensive

- High cost of communication encourages in-network processing

- Limited storage implies all the data generated cannot be stored

- Sensor nodes need to determine how best to allocate its sensing tasks so as to satisfy the queries

- Data is noisy

  - Range queries and probabilistic or approximate queries more appropriate than exact queries

- Classification of Queries

- Long-running vs ad hoc

  - Long-running: Issued once and require periodic updates

  - Ad hoc: Require one-time response

- Temporal:

  - Historical

  - Present

  - Future: e.g., trigger queries

- Nature of query operators

  - Aggregation vs. general

- Spatial vs. non-spatial

- The DB View of Sensor Networks

Traditional

Procedural addressing of individual sensor nodes; user specifies how task executes; data is processed centrally.

DB Approach

Declarative querying and tasking; user isolated from   “how the network works”;        in-network distributed processing.

[TinyDB, Cougar]

- Example queries

- Snapshot queries:

  - What is the concentration of chemical X in the northeast quadrant?
SELECT     AVG(R.concentration)
FROM       Sensordata R
WHERE      R.loc in (50,50,100,100)

- Long-running queries

  - Notify me over the next hour whenever the concentration of chemical X in an area is higher than my security threshold.
SELECT      R.area,AVG(R.concentration)
FROM        Sensordata R
WHERE      R.loc in rectangle
GROUP BY  R.area
HAVING     AVG(R.concentration)>T
DURATION (now,now+3600)
EVERY       10

- Query processing model

- Users pose queries of varying frequencies and durations at gateway

- Queries dispatched every epoch

- Query propagation phase:

  - Queries propagated into the network

- Result propagation phase:

  - Query results propagated up to the gateway in each round of the epoch

- Query propagation

- Construct an aggregation tree

- Propagate query information for epoch along the tree

  - Query vectors, frequencies, durations

- The tree will be used for result propagation during the epoch

- What aggregation tree to construct?

- How should a node schedule its processing, listening, receiving and transmit periods?

  - Can be done on the basis of the level in the aggregation tree

- How should we adapt the aggregation tree to network changes?

- Result propagation

- Given an aggregation tree and query workload, find an energy-efficient result propagation scheme

- In-network processing

  - Sensor network is power (and bandwidth) constrained

  - Local computation is much cheaper than communication